Gian Carlo Trilles

BSCpE-2A

Progress Report: AC/DC Power Supply Project

I. Introduction

This progress report details the development of an AC/DC power supply project. The primary objective is to design, simulate, fabricate, and assemble a power supply unit that converts AC mains power into a stable DC output. The team initiated the process by simulating the circuit design in Multisim to confirm functionality. Following successful simulation, the team advanced to PCB layout, component soldering, and partial enclosure assembly. Throughout the project, emphasis has been placed on accurate PCB design, proper component integration, and tidy assembly within the casing to ensure a functional and safe power supply.

II. Materials Used

• 1 pc Multi-Tapped Transformer (3V, 4.5V, 6V, 9V, and 12V, 750mA)
• 1 pc LM7805 IC Regulator
• 1 pc USB Port
• 1 pc Plastic Case
• 4 pcs 1N4001 Diode
• 1 pc 1000µF / 16V Electrolytic Capacitor
• 1 pc 2200µF / 16V Electrolytic Capacitor
• 1 pc 5Ω 1W Resistor
• 1 pc 680Ω 1/2W Resistor
• 3 pcs 1kΩ 1/4W Resistor
• 5 pcs LED (Green)
• 2 pcs LED (Red)
• 1 pc LED (Orange)
• 1 pc Rotary Switch (2-Pole, 6-Position)
• 2 pcs DPDT Slide Switch
• 1 pc SPDT Switch
• 2 pcs Banana-Type Jack (Red)
• 2 pcs Banana-Type Jack (Black)
• 2 meters Stranded Wire #22 (Black)
• 2 meters Stranded Wire #22 (White)
• 2 meters Stranded Wire #22 (Red)
• 1 pc Fuse Holder with 0.5A Fuse

III. Construction Process

A. Simulation and Design

- The circuit schematic was simulated using Multisim to verify functionality prior to fabrication.

B. PCB Design

- The verified schematic was used to create a PCB layout on graphing paper.

- The layout was transferred onto a copper-clad board.

C. PCB Etching

- The toner transfer method was employed to apply the layout to the board.

- Ferric chloride solution was used to etch away excess copper, leaving the desired circuit traces.

D. Component Soldering

- Holes were drilled into the PCB, and all components were soldered in place.

- Careful soldering techniques were applied to ensure clean, secure connections and prevent short circuits.

E. Enclosure Assembly

- The assembled PCB and transformer were positioned inside the plastic case.

- Parts are currently being aligned and secured within the enclosure.

IV. Current Status & Next Steps

A. Completed

- Multisim simulation

- PCB fabrication and etching

- Component soldering

B. Ongoing

- Placement and securing of parts inside the casing remain incomplete.

C. Next Steps

- Wiring of internal components

- Final testing and troubleshooting to ensure stable voltage output

V. Gdrive link documentation

https://drive.google.com/drive/folders/1ST3FG3BqmxUo7arUStCowCUan4RmOf0V